SeaLife Tech advice

FOR Authorized SeaLife Service Centers and Distributors

Date:			September 21, 200

Subject:	Digital Pro Flash (SL961) - flash does not fire.

Applies to:	All Digital Pro Flashes

Symptoms:	The Flash-Ready light turns on but the flash will not fire when using camera or when switching to TEST. The Flash-Ready light will illuminate immediately after powering on flash (normally it takes a few seconds for the flash-ready light to illuminate after powering on flash).

Cause:	One of the four batteries is damaged or not charged.  There is not enough voltage to operate the internal processor.

Solution:	Use a volt meter or battery charger to confirm that each battery is fully charged.  Each AA battery should have 1.5 volts (for Alkaline) or 1.42 volts (for NiMH rechargeable). If a new alkaline battery or freshly charged NiMH battery does not have this voltage, replace the battery.

Warranty:		No

Repair:	Replace batteries.  If this does not fix the problem, please contact the SeaLife service center in your country.